### Release Notes for the RayTracer Software Suite 3.11.0

The changes made for the 3.11.0:

- Fixed bug in Laser that deleted laserversion.txt when a second instance of Laser was started.

- The function DoLaserCleanup() is now called if a user reboots or shuts-down in RayTracer Operator.

- Fixed a manual clipping bug that can happen if a CAD pt is exactly on the clip boundary (slightly different bug from the one fixed in 3.10.0)

- Replaced “Check Secondary Targets” function in Operator’s ‘Projection’ menu with “Verification Mode” for parts.  This allows part data to be used to describe locations of secondary targets for use in verifying the placement of an attached ‘detail’ or secondary tool. When a part is marked as being a Verification Mode part in Administrator, the user can also specify a non-default location for the results to be logged to (the default is ‘VerificationLog.txt’ in the same directory that Operator is installed into). The CHECK_SECONDARY_TARGETS parameter in Raytracer.INI is obsolete with this change.

The changes made for the 3.10.0:

- Added “Check Secondary Targets” function to Operator.  This allows the expected position of secondary targets (not used for alignment) to be scanned, so the placement of an attached ‘detail’ or secondary tool can be verified.  This function is made available by adding CHECK_SECONDARY_TARGETS=1 to the Raytracer.INI file.

- Fixed a bug in Administrator that is exposed when a tool file is (erroneously) missing the DVAL (drift check) specification in the target definitions.  In this instance, Administrator would not save changes to the drift check status in the target data.

- Made the Focus Dialog come up in the center of the Operator window, and made sure the screen cursor stays in the center of the Focus Dialog.  Also made the laser spot motion smoother.

- Fixed a bug where the error message “the following lasers are not operational” neglected to list the lasers that weren’t operational.

- Fixed a manual clipping bug that can happen if a CAD pt is exactly on the clip boundary.

- Included LPT4 option in the install script and LPT4 support in Laser (including expecting a system type of ‘LPT4’ in the LptSys.ini file).

- Made “display targets” work better on LPT5 projectors (beam was moving too fast).

- Fixed bugs in the Remote API command SetSystemOrientation.

The changes made for the 3.9.0:

- Work-around in Laser for a compiler bug that sometimes caused projections to be truncated, thinking that there were too many points (compiler bug truncation at about 3200 instead of 20000).

- Fixed a bug in Administrator that caused XML files with periods in the name before the extension (e.g. “0.50_thick.xml”) to be parsed incorrectly.

- New modes of Tool Movement test (drift check) have were implemented as follows:

  - Full Test

    - This is the way it has always been done.  If any target is measured as having moved more than the allowed tolerance, or less than 3 targets can be seen (whether the've moved or not), an immediate auto-alignment is started, at the end of which projection resumes.

  - "Full" test with confirmation on failure:

    - If any target is measured as having moved more than the allowed tolerance, a message pops up informing the user that the test has failed and waits for confirmation before starting the auto align.  This gives the user a chance to make sure all the targets are visible to the projector before running the auto-align.

    - If fewer than 3 targets are found (and none have moved more than the allowed tolerance) a message pops up informing the user that there are too few targets visible and waits for confirmation before re-trying the test.  This gives the user a chance to make sure targets are visible to the projector so it can perform the tool movement test.

  - "Limited" test:

    - This is a new mode of test.  If any target is measured as having moved more than the allowed tolerance, projection is stopped and a message pops up waiting for confirmation before starting auto align. This gives the user a chance to make sure all the targets are visible to the projector before running the auto-align.

    - If all the targets found and measured have not moved more than the allowed tolerance (even if it is only one target) the test passes and projection continues.

    - If no targets are visible, projection continues, but a message pops up informing the user that a test was unable to determine if there has been drift and allows the user to re-try the test or just continue.

The changes made for the 3.8.7:

- Fixed a bug in Laser that would sometimes cause focus on LPT1 and LPT10 projectors to drive the focus all the way to the limit switch (instead of a small move) or not move at all, if it was already at the limit switch.

- Fixed a bug in Operator that caused it to sometimes hang and crash when moving the laser via the mouse (alignment or focus). This would only show if the projector’s CPU was heavily loaded during this operation (like going through VNC).

- Fixed a bug in Operator that was causing communication to be lost when moving the laser via the arrow keys (alignment or focus).  This would only happen if the projector’s CPU was heavily loaded during this operation (like using virus checking software).

- Fixed a bug in Operator that left the Laser on when canceling Focus on LPT 5 projectors.

- Fixed a bug in Operator that kept the laser from being controlled via the mouse or arrow keys (alignment or focus) when Operator was running on a separate computer and the projector’s disk was mounted after Operator was started.

- Added Auto Next Layer (Zoom On Next) to Job Details in Operator. Job Details now show when Auto Next Layer is Enabled or Disabled.

- Fixed a bug in Operator that would keep the background screen color white or gray when exiting Downtime Reason. Screen now returns to normal color (red or green).

- Fixed a bug in Operator that caused the instruction display to close when clicking anywhere in the instruction display. Instruction display now closes only when ‘OK’ is clicked. Also, the instruction display no longer takes up the whole screen, and can me moved and manually resized.

- Operator now checks to see if it and Laser are running compatible versions.  If not, it won’t connect.  “Compatible” means the same major and minor versions, but the can be of different revisions (e.g. 3.8.4 and 3.8.5 are compatible but 3.8.4 and 3.7.1 are not)

- Replaced “Network error” with “Communication error” to limit confusion between communication problems between Operator and Laser and actual network errors.

- Fixed a bug in Dot recognition on LPT10 projectors that was causing good scans to be rejected as insufficiently accurate.

- Operator now displays a message instead of hanging when Laser has not been started.

- Fixed a bug in Laser that could cause incorrect target search behavior, in rare occasions.

- A number of fixes to Administrator to remind the user to save changes when closing a window or exiting the application.

The changes made for the 3.8.6:

- Fixed a bug in the remote API for Operator that was not allowing the user to change databases remotely.

- Fixed a bug in the embedded software.  It now disallows ‘zoom’ in an area around the central ‘blind spot’ (where the laser beam reflects internally off the glass window).

NOTE: it is not mandatory to update the embedded software.  The rest of the Raytracer Software suite will work with the previous version (16) of the embedded code.

The changes made for the 3.8.5:

- Fixed a bug in Laser that was causing incorrect results when manual clipping was done in 3 dimensions simultaneously.

- Fixed bug in Laser that was substituting Line feature measurements for Edge feature measurements (as of 3.8.3) -- LPT10 using Verisurf only.

- Some improvement in feature recognition on LPT10 projectors.

- Fixed a bug in Operator that was causing "Invert Normals" to be true if the INVERTNORMALS attribute was not specified in the XML file.  This was happening when we projected parts that had been created from imported .prt files.

- Administrator now correctly handles XML control characters (& < > “ ‘) in ply names by using the correct encoding string for each (e.g. &amp; for & or &lt; for <)

- Operator now automatically starts listening on socket 10101 for remote commands.

- Added the name of the layer to the “projecting chosen layer” log entry.

- Operator now allows Job, Tool, Part and Laser Group to be changed using keyboard access via 'hot-keys'.  The default keys are 'J', 'T', 'P' and 'L' respectively.  These can be changed for different languages via the 'TextStrings.txt' file.   Once focus has passed to a particular pull-down, the desired entry can be selected by highlighting it using the up/down arrow keys and the 'enter' key.  'ESC' will jump out without making a selection.

- The “Job” menu entry (on the top menu bar) is now called “Job Config” to avoid confusion with the “Job” selection control.  Also, the hot key for the Projection menu item is ‘r’ to avoid confusion with the hot key for the “Part” selection control.

- Administrator-level (level 8) users of Operator can now make Operator run in a normal window via the standard control buttons at the top-right of the Operator window.  Other users can still only run Operator as a maximized window.

The changes made for the 3.8.4:

- Fixed a bug in Laser that was causing it to display an LPT10 scan-box momentarily before each target scan after the first 4 in a manual align, which could sometimes cause the scanning to fail.

- LPT10 dot recognition was made faster and more robust.

- The ability to set “Adap = 2” in the LPTSys.ini file (it’s normally set to 1) has been enabled to allow the projections to be somewhat more precise at acute corners (which will cause slightly more flicker).

The changes made for the 3.8.3:

- Fixed a bug in Laser that was causing it to sometimes behave erratically when many commands were sent from Operator in a short period of time.  This was mainly showing itself when users held down an “arrow” key during manual align.

- Fixed a bug in Operator that was causing it to report: “Couldn’t update local database cache” when DowntimeReasons.xml (an optional file) was not present in the network database.

- When the user specifies an existing database on Operator’s command line (via –dbdir), the database path will be used in the login screen, even if the user and password were not specified, or were specified incorrectly.

- Made circular feature recognition (dots/holes) on LPT10 projectors more robust.  It is now better able to handle perturbations in the edge, (e.g. holes that have chips in the edge or lines that intersect the edge of the hole, or specular reflections from the inside of a hole).

- The focus function in Operator on LPT10 no longer does a default auto-focus as soon as the focus point is selected.  Auto-focus is done only when the ‘Auto-Focus’ button is clicked.

- Modified the focus function in Operator to remove confusion that results when a user tries to focus an LPT2 or LPT5, which can only be focused via the knob on the side of the projector.

- ‘ESC’ is now ignored while an auto-focus is in progress on an LPT10.

- Fixed a bug in Operator that was causing it to not recognize database names that contained spaces.

- Fixed a potential infinite loop in Laser (happens only if focus is called when the focus hardware is malfunctioning)

- Added Line feature recognition to the image processing on LPT10 projectors.

- Increased Operator’s timeout for LPT10 scanning operations.  This was mainly a problem for scans done remotely (via Verisurf).

The changes made for the 3.8.2:

- Fixed a bug in Administrator that was causing "Invert Normals" to be true if the INVERTNORMALS attribute was not specified in the XML file.  This was happening when .prt files were imported.

- Changed Auto-clipping to use the projector that is closest to a given CAD point to display that point, rather than the one that has the point closest to its field of view.

- Fixed a bug in Laser that was causing it to hang on LPT5s when the focus screen in Operator was brought up.

- Fixed a bug in Laser that was causing some valid alignments to be rejected after the first 4 targets were scanned.

- Fixed a bug in Operator that was disabling auto-align for some tools with large values in the target coordinates. (>1000).

The changes made for the 3.8.1:

- Fixed a bug in Laser that was causing the LPT10 Scanning Testbed program to report the wrong voltage and gain in the statistics display after scanning extremely bright objects (LPT10 only).

- Made alignment of projectors to extremely small, somewhat inaccurate tools somewhat more precise.

- Fixed a bug in LPT10 auto-focus that caused the focus position to be set higher than the supposed maximum value (LPT10 only).

- Made auto-focus more robust by scanning multiple times when the signal is poor (dark or distant objects) to reduce the effect of electrical noise (LPT10 Only).

- Fixed a resource-leak in Laser that was causing it to crash after 22 alignments.

- Made Operator compatible with projectors running WindowsXP.

The changes made for the 3.8.0:

- Made changes to Raytracer Laser that make alignment significantly more robust and more precise for small tools or tools at significant angles to the projector.

- Administrator Database Cleanup now asks for permission to fix validation errors in XML files that contain extraneous data. Also any XML files listed as containing errors will now report specific error reasons when the selected file is clicked.  This will help with extraneous data that is incorrectly put in the XML files created by some 3rd party applications.

- Administrator user-interface now supports custom language localization from English to another language by using the file, TextStringsAdmin.txt. Users can edit that file for customized language output at their own risk.

- Fixed the focus slider control in Operator so that it accurately reflects the position and range of the focus motor.

- Implemented auto-focus for LPT10 projectors.  This is available in Operator and the Scanning Testbed (TLPScan).

- Fixed a bug in Operator that was leaving the manual and auto alignment buttons active when no job was loaded.

- Fixed a bug that was causing the “pause projection” button to be disabled after an auto-align that was kicked off by a failed drift check.

- Fixed a bug that was causing the target scan to unnecessarily measure and rescan when targets are at an angle to the projector.

The changes made for the 3.7.1:

- Operator user-interface supports custom language localization from English to another language by using the file, TextStrings.txt. Users can edit that file for customized language output at their own risk.

- Fixed a bug that was causing LPT10 projectors to inconsistently set amplification when in “photodiode” (low gain) mode. Bug was introduced at 3.7.0

The changes made for the 3.7.0:

- Added drop down selection menu in Administrator to allow feature type change for existing targets.

- Added a “Reuse Tool Alignment” checkbox option to the Job Parameters dialog in Administrator. When the Reuse Tool Alignment option is enabled for a job, Operator caches the tool target alignment data after a successful manual alignment. Once the data is cached, auto-align is enabled for other tools as long as the new tool contains the same number of targets and same XYZ (or relative XYZ) positions as the previous tool.

- Fixed an access violation caused by projections with too many (>20K) pts.

- Fixed an access violation caused by projections with greater than 500 part layers.

- Added Get/Set System Orientation functions to Java API support.  Java API interface is now 1.2

- Added suppression of specular reflections (small shiny areas reflecting lots of light back) to the LPT10 feature recognition so that shiny areas can be scanned and processed in a more robust way.

- Changes to Laser to improve speed and fix some potential access violations and memory leaks.

- Some fixes for Verisurf measurement tools.

- API fix – if an auto align is started by drift check, the event notification is sent with the ‘asynchronous event’ flag set.

The changes made for the 3.6.2:

- Added functionality to allow the user to specify which job should be automatically loaded once Operator attaches to the database by specifying the name of the job on the command line as: ‘-job’ or ‘/job’ <job name>

- Added functionality to let Operator optionally start listening on a socket automatically at start up by specifying the socket number on the command line as: ‘-socket’ or ‘/socket’ <socket #>

- Fixed an access violation caused by projections with too many (>20K) pts.

- Keep the focus window in Operator from being accidentally resized.

- Keep the mouse cursor closer to the center of the screen so it stays on the focus window.  This way no mouse clicks will be missed.

- Fixed a bug in Operator that caused inconsistent responsiveness when positioning the laser during manual alignment.

- Enhanced Administrator menu functions, ‘Database Cleanup’ and ‘Copy XML file into database’ to display filenames containing XML parsing (ill-formed) or validation (existing data) errors and where moved (to Extraneous folder).

- Added ‘Assist’ functionality to the Scanning Testbed (TLPScan).

- Added ‘Edge’ (linear edge) and ‘Raw Contour’ (freeform contour) features to the feature recognition library for use by the Verisurf Measure / Reverse-Engineering tool.

- Added ‘Edge’ and ‘Raw Contour’ to the Scanning Testbed.

- The Scanning Testbed now determines whether a feature was found with sufficient accuracy to be considered ‘good’.

- The “License expired” message now gives the correct phone number.

- Fixed a bug that let malformed tool and part files to cause Administrator to exit.

The changes made for the 3.6.1:

- Fixed a problem that was causing targets to be unnecessarily scanned a second time.

- Long database path names were not scrolling in the Administrator and Operator login screens.

- Enforced 10% whitespace around the target image during display.

- LPT10 feature recognition improvements – will more often recognize low-contrast images.

The changes made for the 3.6.0:

- Significantly faster searching and scanning of retro targets for LPT1, LPT2 and LPT5 projectors.

- Faster startup of Operator.

- Added “assist” functionality for Dot features (LPT10).

- Feature recognition is now more robust, with better precision (LPT10).

- Made the Scanning Testbed (TLPScan) keep the mouse cursor stationary while the scan cursor (displayed by the laser) is being moved around (LPT10).  This keeps the cursor from sliding off the application window.

The changes made for the 3.5.3:

- Fixed a bug in Administrator that was causing Add/Insert Row buttons to have no effect, only the arrow button next to the Add/Insert buttons.  Clicking on add/insert row in the tool menu will now add/insert a default target to the tool.  If the user wants to add/insert a feature, they do this by selecting the pull-down menu next to the ‘add’ or ‘insert’ buttons.

- In Administrator, the default metric Parameter Set had the values for the individual and RMS target misalignment limits reversed.

- Administrator can now import multiple .XML files (like tool or part files) at one time.

- In Administrator, the minimum individual target drift limit was changed from 0.05 to 0.005

- Fix bug in Administrator that was causing invalid tools to be created if the user added targets, then changed the targets’ laser numbers, then did a save.

- Made Laser more accurate when finding targets during a manual align (after first 4 targets).

- Fixed a potential access violation in Laser (in the alignment code).

- Fixed a bug in Laser that was causing it to claim a good alignment when, in fact, the alignment had failed (usually due to too low a signal/noise ratio in target positions).

- No longer put an icon for diagnostics on the desktop when Raytracer Laser is installed.

- Fixed a few bugs in Operator that caused it to incorrectly go into an “unaligned” state, or to show an “aligned” (green) screen when it was actually “unaligned”.

- Changed “Local Database” to “Local Database Cache” in Operator to avoid confusion between our local cache and a networked database that happens to be on the local disk.

- “Auto Alignment” button in Operator will be disabled (greyed-out) when manual align is needed.

- Removed the requirement of rebooting after installing the RayTracer Software.

The changes made for the 3.5.2:

- If the previously accessed database is no longer accessible, show the user a message to that effect, so they won’t assume that they’re logging into that database, only to find themselves in the ‘local database’

- Operator and Administrator login screens made more intuitive – database being accessed is more obvious.  Logging into a different database from the main program automatically starts up a file browser.

- Fixed a bug in Operator that was causing auto-align to fail in the absence of a DowntimeReasons.xml file.

- Fixed a bug in Administrator that was causing multi-projector tools (used for clipping) to have invalid data (target numbers were being generated per-tool, rather than per-laser)

- Fixed a bug in Administrator that was causing features (but not targets) to sometimes update or redisplay incorrectly.

- Fixed a buffer overrun in Laser that was taking place when processing a ply that had more than 10000 points in it.

- Fixed a bug in Laser that was causing occasional disconnects due to a race condition.

- Removed a potential infinite loop in Laser’s target measurement function.

- Scan cursor on LPT10 is now much more responsive when being controlled from Operator running on a different computer/projector.

- “Dot” feature recognition on LPT10 projectors is somewhat more robust now.

- Increased part file parsing.  No longer parse the part a second time to add offsets unless the offset is non-zero.

The changes made for the 3.5.1:

- Changed Operator to allow any users to shut down remote projectors, instead of only Administrators.  This is enabled only when the “Operator may shutdown” flag is set in a job.

- Administrator “Copy from Raytracer DB…” was changed to “Copy XML file into database…” to help avoid confusion about what it does.

- TLPScan is now referred to as ScanTest or Scanning Testbed and will now allow the user to scroll the scan if it’s too big.

- Fixed a bug in LPT10 diagnostic that caused upgrading the embedded code to fail.

The changes made for the 3.5.0:

- Full support of LPT10 feature recognition for single-point “dots” and “intersections”.  This includes a change to the Tool GUI of Administrator to allow for the creation of targets, dots or intersections in a tool, rather than just targets.

- Support for installing Raytracer Laser on LPT10 projectors.

- An “assist” function was added for showing the image processing routines which of several potential intersections is the intersection of interest in a scan.

- Some minor changes to the projector alignment GUI to accommodate the “assist” function.

- The inclusion, on LPT10 projectors of “TLPScan” – a simple program for scanning and processing features outside of Operator.  This is intended for use in planning tools that contain features.  An icon for this program is created on the desktop and in the Raytracer Laser icon group in the Start menu.

- Added the SystemID program to the Laser Operator icon group in the Start menu, for use in getting information necessary for obtaining a software key.

3.4.0 – internal release only.

The changes made for the 3.3.0:

- Fixed a bug that was causing targets to be displayed at different sizes sometimes. This only affected how the image of the target is shown to the user, the calculation for the center of the target was/is correct.

- Was saying “Target in blind spot Target too small” instead of a reasonable explanation of the target being too large or too close to the center of the FOV.  Fixed now.

- API for remote control of Operator was implemented.

- Image Record and API are now locked/unlocked using the same license system as Operator itself.

- The laser cursor motion (in focus and alignment) is now non-linear so that smaller mouse motions cause a finer movement of the laser spot and larger motions a coarser movement.

The changes made for the 3.2.2:

- More efficient algorithm for finding the target.

- Fixed an intermittent (rare) bug that was causing a spurious target to be “found” and a resulting blank scan and “target missed” message.

- Made the system report if it saw a target that was too small (approx .02 inches at 15 ft).

The changes made for the 3.2.1:

- Round the position of the cross-hair displayed on the target image correctly.

- Fix a potential infinite loop in the target bounding box measurement code.

- Mouse control of the laser is now more responsive for small movements

The changes made for the 3.2.0:

- Laser now does a timing test on startup (takes approx 0.6-1.0 seconds) and uses the results to ensure subsequent projectors with potentially faster CPUs will still accurately find and measure the targets.

- Properly reset thread priority when exiting from the target bounding box measurement routine.

- Fixed a bug that caused an infinite loop in Laser if a target intersected the Field of View.

The changes made for the 3.2.0 BETA:

- CPU no longer runs at 100% during auto-alignment.

- External sign is blanked when Operator exits.

- Administrator now displays any problems it finds while importing tools or parts.

- Changed “Done” checkbox in Administrator Import function to a simple text string that is either empty or “Done”.

- Fixed a bug in Administrator that was causing it to reset the Manual Clip Set that a Parameter Set pointed to.

- Target searching and scanning has been sped up (now takes approx half the time)

- Target scanning now automatically calculates the necessary step size for each target.

- Added a fixed scanning resolution mode that keeps step-size constant and calculates the necessary sample size for each target.  This is controlled by setting FixedScanResolution=1 in LptSys.ini, and uses the ScanStepSize as the fixed resolution.

- Annular targets are now supported.

- Target search algorithm changed.  It now uses a two-pass, fixed resolution search (first at step size 3, then if target isn’t found another pass at step size 2).

The changes made for 3.1.6:

- Image Recording: Operator now waits for all projectors to actually begin projecting before starting the exposure timer.

- Operator now copies only files that have newer timestamps from the remote database when the job is reloaded.  This will allow auto-align to be available more consistently.  It will also make reload times faster when dealing with large databases.

- Rework of Administrator’s database access.  There is no longer any need to do database cleanups after making changes via Administrator.  The only time a database cleanup command is needed is if changes are made to the database without using Administrator.

- Made Administrator much more responsive – only refreshing those parts of the database and application graphics that were affected by a particular change, instead of refreshing the entire application.  This will allow Administrator to be much more responsive when accessing databases that are large, or across slow networks (or both).

- Bug fix in Operator where displaying a projection in slow motion would turn drift-check off until the user stopped and re-projected the layer.

The changes made for 3.1.5:

- Auto clipping and Normal clipping weren’t taking the projector’s exchange coordinate system into account, causing errors when projectors were in different coordinate systems.  This has been fixed.

- Changed Auto clipping so that configurations of 3 or more projectors will have ‘false positives’ (multiple projectors displaying a segment) rather than ‘false negatives’ (no projectors displaying a segment).

- Normal clipping now has a Field of View check.

- Fixed a bug in Administrator that was causing all the icons to disappear after a parameter set was saved.

- Fixed a bug in Administrator that caused the pulldown list of clip sets to be blank for a newly created parameter set.

The changes made for 3.1.4:

- Fixed a bug that was causing the enabling of Image Recording to be impermanent.

- Added a control to the Parameter Set screen in Administrator allowing the user to specify what the Max Incident Angle should be for Normal Clipping.

- Made the external sign only use one color and one size of text, and only scroll if the text won’t fit on sign all at once.

- Added functionality to allow user login via the command line on Operator.    The user, password and database can be specified on the command line as follows:

'-user' or '/user' <user>

'-passwd' or '/passwd' <password>

'-dbdir' or '/dbdir' <data base directory path>

If the login fails, the normal login screen will be popped up with the specified values seeded in the user, password and database controls.  If 'dbdir' isn't specified, Operator will try to connect to the last database used.

- Fixed a bug that caused auto-align to malfunction when one of the targets was occluded.

The changes made for 3.1.3:

- Fixed a bug that caused mouse movement multipliers to sometimes be saved as zero.

- Verification after an import is now much faster (only verifying those files that were imported)

- During an import, the cursor is updated to indicate that processing is being performed.  The message that indicated that the import has completed has been delayed until it is complete.

- When Import | Display Log is selected and the log is empty, the string "There are no entries in the log." is displayed.

The changes made for 3.1.2:

- Bug fixed that was causing centerfield cross to not display when event logging was disabled.

- Focus cross will no longer wrap if it intersects the field of view.

- For parts that have layer-by-laser enabled, when marking a layer complete, Operator wasn't recognizing that it was currently projecting when it actually was.

The changes made for 3.1.1:

- “Zoom on Next” is now referred to as “Auto Next Layer”.

- Drift Check is no longer disabled by enabling “Auto Next Layer” or turning on “Zoom”

- The External Sign (marquee) functionality has been put back in.  The feature is enabled through Administrator, via the Parameter Set screen.  When a job using the external sign is loaded Operator will look for the sign for 1.5 seconds.  If the sign is not attached, Operator will show a message to that effect saying that the sign will be disabled.  If the user wants to use the sign after that, they need to check the connections and then reload the job.

- The external sign will display the names of the plies and/or layers being projected.

- The names of the selected plies and layers are now consistently shown in a bold font.

- Inaccuracies in the drift-check measurement have been fixed.

- Focus control through Operator will now use the limit switches to ensure that the lens housing doesn’t move too far to one side or the other.

- Bug fixed that was causing access violations and other non-deterministic problems when trying to apply clipping to a zero-length list of points.

- RTL, Administrator and Operator are all set to Version 3.1.1

The changes made for 3.1.0:

- “Image Recording” functionality was added.  The new parameter (and its default) that can be specified in LPTSys.ini for Image Recording is as follows:

MaterialThresholdIR = 150

- In Administrator, “Quick Cleanup” and “Detailed Cleanup” were replaced with “Database Cleanup”

- “Display Targets” now shows better looking ‘+’ markers.

- Target display and target scan will no longer wrap if they intersect the edge of the field of view.

- “text on/off” and “zoom on/off” are now remain consistent with the actual state the system is in.

- Added the display name of the projector to the alignment screen.

- Double clicking on the current tool sometimes displayed the targets for the first tool in the list instead of the selected tool.  This has been fixed.

- Fixed bug that was causing layer names containing dashes (‘-‘) to be displayed with truncated names.

The changes made for 3.0.4:

- Operator – some cosmetic cleanup, mostly in the Alignment screen.

- Operator – some UI fixes: focus and alignment screens now control the beam in the same manner.

- Database loading - index file problem corrected (in Operator).

- Database: no longer uses the TOOLGALVOS directory (was redundant).

The changes made for 3.0.3:

- RTL and Diagnostics - Added Target Search box size control (per Wichita request) via LPTSys.ini.

- Versioning for RTL and Diagnostics corrected.  RTL version number is set to 3.0.3

- Operator - some UI and cleanups.  Laser beam location control during focusing mode by arrow keys.

- Embedded changed to v.16 - Added "IF DebugOnOff" test around open print statements in Zoom-On-Next sections.

The changes made for 3.0.2:

- RTL and Diagnostics - Corrected Target Scan and PlusCheck.  Eliminated D-shaped target scan images.  Expanded range of acceptable target sizes by introducing self-centering scan of larger targets (secondary scan as needed).

- RTL version number is set to 3.0.0.2

- Embedded changed to v.15 - Revision command is corrected.

The changes made for 3.0.1:

- RTL and Operator - "Zoom-On-Next" is fixed to behave the same way as the NEXT button on the Operator screen.

- RTL version number is set to 3.0.0.1

- Embedded changed to v.14 - Added Zoom Ratio (1 to 100%) feature. Changed Text section to display both Text and Layer data at the same time when Text is ON.