Upgrade of the LPT RayTracer Software Suite from versions 2.18 and later, to version 3.11.1

Date:  February 18, 2009

_______________________________________________________________________

This document describes steps of upgrading the LPT RayTracer Software Suite from versions 2.18 or 3.0.0 and later, to version 3.11.1 on an LPT projector using the Installation CD.  The LPT projector has to run Windows 2000 operating system and it has to be connected through a network to an external computer that has CD drive.  You have to be logged on as an Administrator or a user with administrative rights on the LPT projector’s operating system.

Installation files are to be copied from the CD to the projector’s hard drive via the network.  Alternatively, all installation files and folders from the Installation CD can be copied to a USB Flash Drive on any external computer under Windows XP or Windows 2000.  Then, the USB flash drive can be attached to the LPT projector’s USB interface for copying files to the projector’s hard drive.

First of all, create an operating system account for user “Admin” with administrative rights (has to belong to the group of administrators) and password “Admin”.  Make sure that the password is set with “Never Expires” option checked.  For the steps described below you have to be logged to the projector’s Windows operating system as “Administrator” or as “Admin”.

The steps are as follows:

- On the LPT projector’s hard drive create archive folder  D:\Archive_upgrade_to_3.11.1

- Back up the following files and folders by copying them to the created archive folder:

  - Copy file C:\WINNT\LptSys.ini

  - Copy file C:\Program Files\LPT\RayTracer Operator\RayTracer.ini renaming it as RayTracer_oper.ini.

  - Copy file C:\Program Files\LPT\RayTracer Administrator\RayTracer.ini renaming it as RayTracer_admin.ini.

  - Copy folder  C:\GalvoDrivers

  - Copy Folder  C:\NdYag

- Manually move old copies of installation files and the Embedded folder (likely from D:\backups, or a subdirectory thereof if they exist) to the archive folder created in Step 1  -  save these files in case the upgrade fails and a roll back is necessary:

  - Move file  AdminSetup.exe

  - Move file  LaserSetup.exe

  - Move file  OperatorSetup.exe

  - Move folder Embedded.

- Close all running LPT programs:

  - RayTracer Operator.

  - RayTracer Administrator.

  - RayTracer Laser (usually running minimized).

  - Program Manager, (if it’s there, it’s usually running minimized).

- Uninstall previous version of the RayTracer Software Suite software components via Control Panel (Start > Settings > Control Panel > Add/Remove Programs)

  - RayTracer Operator

  - RayTracer Administrator

  - RayTracer Laser

- Manually copy new versions of RayTracer installation files from the Installation CD via network or a USB Flash drive  to D:\backups (create directory if necessary) in the following way:

  - Copy the whole folder V3.11.1 from the CD to D:\backups.

  - For the LPT1 or LPT1XL  projector leave only subfolder

D:\backups\V3.11.1\Embedded\LPT1_V17.1 (you will not need \LPT2_V17.2_5 or LPT10_V17.3).

  - For the LPT2 or LPT5  projector leave only subfolder

D:\backups\V3.11.1\Embedded\LPT2_5_V17.2 (you will not need \LPT1_V17.1 or LPT10_V17.3).

  - For the LPT10 projector leave only subfolder

D:\backups\V3.11.1\Embedded\LPT10_V17.3 (you will not need \LPT1_V17.1 or LPT2_5_V17.2).

- Install new version 3.11.1 of the RayTracer Software Suite as required (not all projectors will have Administrator, and Administrator may be running from locations other than projectors. It is suggested that all instances of Administrator must be upgraded.    To install all the components of the Software Suite onto the projector’s computer run the new executable setup files from D:\backups\V3.11.1  in the following sequence:

  - RayTracer Administrator.

  - RayTracer Laser.

  - RayTracer Operator.

- Perform the embedded processor firmware upgrade for the LPT projector:

NOTE: this step is only necessary when updating from versions previous to 3.0.4

  - Exit RayTracer Laser by closing the Raytracer Laser window.

  - Start the Diagnostics program from the ‘Start’ menu:

(Start > Program Files > LPT > RayTracer Laser > Diagnostics)

  - Click ResetEC several times, wait a couple of seconds between each click.

  - Look for the PC104 Upgrade Section in the upper right corner of the Diagnostics screen.

  - Click in the empty box next the word “passwd” and type in lpt .

  - Click in the empty box next to the word “path” and type in the absolute path to the new lptdisp.exe file, including the file name. Example for the LPT1 or LPT1XL projector:  D:\backups\V3.11.1\Embedded\LPT1_V17.1\LptDisp.exe.

  - Click the button that has “Emb Upg” on it.

  - At the bottom of this section a count should begin incrementing within a few seconds showing the amount of data transferred.

  - If it does not, close the diagnostics screen (this may require a "ctrl-alt-del" or task manager) and try again from the start.  If it fails again completely reboot the LPT projector (this includes a complete power off and on) and try again.  If it fails again – call the LPT factory – a hardware upgrade may necessary.

  - After the upgrade completes click on “Initialize”.  Run “Center Cross” and “Field of View” to verify correct functionality.  Exit the Diagnostics program.

  - Shut down the projector and turn it off for about 10 seconds.  Then turn it back on and let it reboot.

- In the KernelDriver1.2 directory, run the LPT_Driver_Install.EXE program to install version 1.2 of the kernel driver.  Do this on LPT1, LPT2 and LPT5 projectors only.  LPT10 projectors were all shipped with this kernel driver.  This step will necessitate a reboot.

NOTE: this step is only necessary when updating from versions previous to 3.6.0

- If you haven’t rebooted, manually start the newly installed Raytracer Laser by clicking on the desktop icon.

- Run the newly installed RayTracer Operator 3.11.1.  If it requires you to enter the Software Protection Key, run SystemID from the Start>Programs>LPT>Laser Operator menu.  This will give you an 8-digit code.  Call the LPT factory with this code and you will be given a key.

RayTracer Laser / RayTracer Operator compatibility issues:

RayTracer Laser 3.11.1 is not compatible with previous versions of RayTracer Operator and vice-versa.  When upgrading the RayTracer Software Suite for a projection system with multiple LPT heads in master/slave configuration it is necessary to upgrade software in all of the heads at once.

Database compatibility issues:

Compared to versions previous to 3.0, Software Suite version 3.11.1 has some new features added:  “Auto-Next-Layer”, “Remember Alignment”, “Semi-sequential Projection”, and others.  Those new features are being configured in the process of job creation/editing using RayTracer Administrator version 3.11.1.  Settings for the new features are being stored in appropriate fields of the XML-format files for jobs, parameter sets, etc. - components of a customer database.

A database that has some of new features’ components configured using RayTracer Administrator version 3.11.1 is defined as Database “3.0”.   A database that contains components (files) with only previous versions’ features is defined as Database “2.18”.

RayTracer Operator 3.11.1 is OK to work with both database “3.0” or database “2.18”.  With database “3.0” it will properly use all of new features’ settings contained in the database files.  With database “2.18” it will run using default (disabling) settings for the new features thus delivering the same functionality as the RayTracer Operator 2.18.

RayTracer Administrator 3.11.1 is also OK to work with both database “3.0” or database “2.18”.  With database “3.0” it provides full access and editing capability for all the features - new or old.  When used to edit or save some components (jobs or parameter sets) of the database “2.18” the RayTracer Administrator 3.11.1 will change them adding new features’ settings thus elevating the database level to database “3.0”.  Other components of the database “2.18” like parts and tools will be edited and saved by the RayTracer Administrator 3.11.1 the same way as the RayTracer Administrator 2.18 would do.  If only parts or tools editing has been done then the database level stays as database “2.18”.

RayTracer Operator 2.18 is OK to work with database “2.18” and it is compatible with database “3.0”.  It will run database “3.0” the same way as database “2.18” ignoring new features configured in the database “3.0” files.

RayTracer Administrator 2.18 is OK to work with database “2.18” but it is not compatible with database “3.0”.  When used to edit or save some components (jobs or parameter sets) of the database “3.0” the RayTracer Administrator 2.18 will change them erasing new features’ settings from the database thus downgrading the database level to database “2.18” and loosing information.

Summary:

It is recommended that customer, at some point in time, should upgrade all instances of RayTracer Administrator (all locations - projectors and/or remote computers) to new version 3.11.1 even though not all of the LPT projector systems might have been chosen for a full upgrade to the RayTracer Software Suite 3.11.1 at once.

Customers do not need to do anything immediately with existing databases “2.18” at the time of upgrading the RayTracer Software Suite to the new version 3.11.1 on chosen LPT projectors/computes:  the new Software Suite will utilize a database “2.18” in exactly the same way as the Software Suite 2.18.

After upgrading, customers should use only new RayTracer Administrator 3.11.1 to create new jobs with new features or to modify existing ones within any databases as needed.  Gradually, some of databases “2.18” will be modified and saved with new feature fields, hence acquiring a level of database “3.0”.  All databases, old or new (containing only “2.18” features or both “2.18” and “3.0” features), modified or not, will be used in two ways:

- With LPT projectors that are running under RayTracer Operator 3.11.1: automatically utilizing new features if they were configured in database files.

- With LPT projectors that are still running under RayTracer Operator 2.18: automatically ignoring new features even though they were configured in database files.